Data logging/ data uploading / data pushing to Thingspeak server

Data= humidity and temperature of Environment senced by DHT11 sensor

Components required:

- Esp32

- Dht11 sensor

- Jumper wires

- Thingspeak account

- Micro usb cable (data cable type-b)

Circuit Diagram:

Steps:

Create an account in Thingspeak server/platform.

Log in to the platform.

Click on the new channel ;

Create a channel with 2 fields;

Field one = temperature

Field two = humidity

The fields as follows;

Go to API keys and copy the write/read key;

We take write key : to push the data for the environment to server

And read key : to collect the data from the server.

Use the keys in the code to connect the thingspeak server and board.

after that go to devices and select MQTT if u r using MQTT logic(pgm), create device by name dht11.

Next we will get client id , username, and password . use the in MQTT program.

The device of name dht11 is created.

Program:

Note: by using MQTT.

Program is done in the mu editor / tonney in python language.

from machine import Pin

from dht import DHT11

import network

from time import sleep

import sys

from umqtt.simple import MQTTClient

d = DHT11(Pin(15))

SSID = 'WiFi Name'

PASS = 'WiFi Password'

CLIENT_ID = 'ThingSpeak MQTT Client_ID'

SERVER = 'mqtt3.thingspeak.com'

USERNAME = 'ThingSpeak MQTT Username'

PASSWORD = 'ThingSpeak MQTT Password'

client = MQTTClient(client_id = CLIENT_ID,

server = SERVER,

user = USERNAME,

password = PASSWORD)

CHANNEL_ID = 'ThingSpeak Channel_ID'

topic = 'channels/' + CHANNEL_ID + '/publish'

topic = bytes(topic,'utf-8')

def dhtData():

d.measure()

t = d.temperature()

h = d.humidity()

return t,h

def connectWifi():

wifi = network.WLAN(network.STA_IF)

wifi.active(False)

sleep(0.5)

wifi.active(True)

wifi.connect(SSID, PASS)

sleep(2)

if (wifi.isconnected()):

print('Connected')

else:

print('Not Connected')

sys.exit()

connectWifi()

try:

client.connect()

except:

print('Not Connected to MQTT Broker')

sys.exit()

while True:

temp, hum = dhtData()

msg = '&field1={}&field2={}'.format(temp,hum)

msg = bytes(msg, 'utf-8')

client.publish(topic,msg)

print('Temperature=',temp)

print('Humidity=', hum)

sleep(20)

(OR)

Program:

Note: by using Request.

Program is done in the mu editor / tonney in python language.

from machine import Pin

from dht import DHT11

import network

from time import sleep

import sys

import urequests

d = DHT11(Pin(15))

SSID = 'WiFi Name'

PASS = 'WiFi Password'

WRITE_API_KEY = 'Write_API_Key'

def dhtData():

d.measure()

t = d.temperature()

h = d.humidity()

return t,h

def connectWifi():

wifi = network.WLAN(network.STA_IF)

wifi.active(False)

sleep(0.5)

wifi.active(True)

wifi.connect(SSID, PASS)

sleep(2)

if (wifi.isconnected()):

print('Connected')

else:

print('Not Connected')

sys.exit()

connectWifi()

while True:

temp, hum = dhtData()

data = '&field1={}&field2={}'.format(temp,hum)

request = urequests.get('https://api.thingspeak.com/update?api_key=' + WRITE_API_KEY + data)

request.close()

print('Temperature=',temp)

print('Humidity=', hum)

sleep(20)

Output:

||